GPS手表通信协议

目录

1. 协议格式 .............................................................................................................................. 3

2.  协议规范 ............................................................................................................................... 4

1、心跳包 (0x0001) ............................................................................................................ 4

2、请求服务器动态分配地址 (0x0002) .............................................................................. 4

3、单次地址查询 (0x4101) ................................................................................................. 4

5、设置亲情号码  (0x4103)................................................................................................ 5

6、设置监听号码 (0x4130) ................................................................................................. 5

7、设置时区 (0x4132)......................................................................................................... 6

8、查询亲情号码(0x9003) .................................................................................................. 6

9、定位信息上报 (0x9955/0x9956) ................................................................................. 7

10、短信转发(0x9957) ....................................................................................................... 7

11、请求中文地址信息(0x9958) ........................................................................................ 8

3. 附录1: 0x9955数据格式 ................................................................................................ 9

1.  协议格式

GPRS数据包使用TCP传输，协议格式定义如下： 服务器下发到终端设备（下行） @@<L><ID><协议号><数据><checksum>\r\n 终端设备上报到服务器（上行） $$<L><ID><协议号><数据><checksum>\r\n

| 协议字段 | 字节 | 格式说明 |
| --- | --- | --- |
| @@ | 2 | 服务器下发到终端设备起始位，字符@十六进制编码为 0x40 |
| $$ | 2 | 终端设备上报到服务器始位，字符$十六进制编码为0x24 |
| L | 2 | 包长度，整个数据包长度的十六进制 |
| ID | 7 | 设备ID 14位的BCD编码，如设备号为01234567890123编码后的7字节数据为0x01,0x23,0x45, 0x67, 0x89, 0x01, 0x23 |
| 协议号 | 2 | 参见下章数据包描述 |
| 数据 | N | 支持0-100字节 |
| checksum | 2 | 数据包的CRC-CCITT (Kermit)校验位，不包括校验位本身和数据包结束标记位 如24240011300700000005710001 42ED 0D0A 42ED = CRC (24240011300700000005710001) |
| \r\n | 2 | 包结束符，十六进制表示为0x0D 0x0A |

注意： 1、“<”和“>”仅做数据包分隔标记，不是数据本身；多字节数据编码为大端模式。 2、ID取设备IMEI号前14位，使用十六进制为7个字节。 3、终端设备上报数据包到服务器定义为上行，服务器下发数据包到终端设备定义为下行。 4、描述中“请求”和“上报”表示终端主动向服务器发起数据包，“设置”和“查询”表示服务器主动发数据配置参数，“响应”表示被动答复对方的数据包。 5、为避免重复特此说明，后续不再描述，协议包中的<Flag>表示1个字节的数据，其含义为1表示操作成功，0表示操作失败。 6、范例中数据均为16进制表示。 7、该文档为精简版，仅保证实现基本功能，部分协议未录入。

2.协议规范

1、心跳包 (0x0001)

| 功能 | 终端定时上报心跳包，确认GPRS网路连接正常，否则终端再次重连 |
| --- | --- |
| 上行 | $$<L><ID><0x0001><checksum>\r\n |
| 下行 | @@<L><ID><0x0001><Flag><checksum>\r\n |
| 范例 | 2424 0011 30020000000013 0001 0B1D 0D0A |
|  | 4040 0012 30020000000013 0001 01 F179 0D0A |

| 功能 | 终端请求服务器动态分配地址，并连接新的地址，实现服务器负载均衡 |
| --- | --- |
| 上行 | $$<L><ID><0x0002><checksum>\r\n |
| 下行 | @@<L><ID><0x0002 ><IP:port><checksum>\r\n |
| 范例 | 4040 0023 30020000000013 0002 3130362E332E3232362E3138303A38303930 73C2 0D0A |

2、请求服务器动态分配地址 (0x0002)

表示终端需要连接服务器106.3.226.180:8090，开始正常工作。

3、单次地址查询 (0x4101)

| 功能 | 服务器查询设备当前位置 |
| --- | --- |
| 下行 | @@<L><ID><0x4101><checksum>\r\n |
| 下行 | $$<L><ID><0x9955><data><checksum>\r\n |
| 范例 |  |

4、设置定位包上传时间间隔(0x4102)

| 功能 | 设置终端定位包0x9955的上报间隔 |
| --- | --- |
| 下行 | @@<L><ID><0x4102>< interval in hex code (2 bytes)><checksum>\r\n |
| 上行 | $$<L><ID><0x5100><Flag> <interval (2 bytes)><checksum>\r\n |
| 范例 | 4040 0013 30020000000013 4102 000A 3808 0D0A |
|  | 上行缺少 |

注意：时间间隔为0表示停止向服务器上报定位数据。

5、设置亲情号码  (0x4103)

| 功能 | 设置终端亲情号码 |
| --- | --- |
| 下行 | @@<L><ID><0x4103><button number>< phone number for receiving SMS>< phone number for receiving call><checksum>\r\n |
| 上行 | $$<L><ID><0x4103><Flag><checksum>\r\n |
| 范例 | 4040 0032 30060000000007 4103 01 31313031313031313000000000000000 31313031313031313000000000000000 B1DA 0D0A |
|  | 24240012300600000000074103000D8D0D0A |

注意： 1、Button number – 1个字节16进制的数据，支持4个，包括SOS按键和3个速拨按键。

| button number | 对应按键 |
| --- | --- |
| 1 | SOS button |
| 2 | button 1 |
| 3 | button 2 |
| 4 | button 3 |

2、号码使用16位ASCII表示，不足16位用0补充，本协议中所有电话号码均按此规范。如号码 8613612345678表示为 0x38 0x36 0x31 0x33 0x36 0x31 0x32 0x33 0x34 0x35 0x36 0x37 0x38 0x00 0x00 0x00。

6、设置监听号码 (0x4130)

| 功能 | 设置监听号码 |
| --- | --- |
| 下行 | @@<L><ID><0x4130><data><checksum>\r\n |
| 上行 | $$<L><ID><0x4130><Flag><checksum>\r\n |
| 范例 | 4040 001E 30060000000007 4130 31323334353637383930313233 3276 0D0A |
|  | 上行缺少 |

7、设置时区 (0x4132)

| 功能 | 设置终端时区 |
| --- | --- |
| 下行 | @@<L><ID><0x4132><data><checksum>\r\n |
| 上行 | $$<L><ID><0x4132><Flag><checksum>\r\n |
| 范例 |  |

注意：部分项目可能会导致异常。

8、查询亲情号码(0x9003)

| 功能 | 服务器查询终端绑定的亲情号码 |
| --- | --- |
| 下行 | @@<L><ID><0x9003><button number><checksum>\r\n |
| 上行 | $$<L><ID><0x9003>< phone number for receiving SMS><phone number for receiving call><checksum>\r\n |
| 范例 | 4040 0012 30060000000007 9003 04 92B5 0D0A |
|  | 2424 002130060000000007 9003 30303030303030303030303030303030 3118 0D0A |

| button number | 对应号码 |
| --- | --- |
| 1 | SOS button |
| 2 | button 1 |
| 3 | button 2 |
| 4 | button 3 |
| 0xFF | 查询全部号码 |

9、定位信息上报 (0x9955/0x9956)

| 功能 | 终端上报位置信息 |
| --- | --- |
| 上行 | $$<L><ID><0x9955><data><checksum>\r\n |
|  | $$<L><ID><0x9956><data;data;data;data;data;data;data;…data;data;data;><checksum>\r\n |
| 下行 | @@<L><ID><0x9955 ><Flag><checksum>\r\n |
|  | @@<L><ID><0x9956 ><Flag><checksum>\r\n |
| 范例 | 2424 0076 30060000000007 9955 3034353635312E3030302C412C323233322E323336352C4E2C31313430312E333738382C452C3030302E312C3135372E35372C3132303931342C2C7C31307C3130307C343638302C31303137332C3030302C3436307C303030307C30307C3036367C303932 1CC0 0D0A |
|  | 4040 0012 30060000000007 9955 01 DE21 0D0A |

注意：

- 1、 范例0x9955包中对应ASCII为045651.000,A,2232.2365,N,11401.3788,E,000.1,157.57,120914,,|10|100|4680,10173,000,460|0000|00|066|092

具体格式见附录1。 2、0x9956是将多条0x9955包合并，中间使用“;”分割，其中data最多40项。

10、短信转发(0x9957)

| 功能 | 服务器下发电话号码和短信内容，终端将其转发 |
| --- | --- |
| 下行 | @@<L><ID><0x9957><phone number><SMS data><checksum>\r\n |
| 上行 | $$<L><ID><0x9957><Flag><checksum>\r\n |
| 范例 |  |

注意： 1、“phone number”为16位的ASCII电话号码。 2、“SMS data”为大段unicode码。 3、格式参考其他数据包。

11、请求中文地址信息(0x9958)

| 功能 | 终端上报位置信息，请求服务器转为地址描述 |
| --- | --- |
| 上行 | $$<L><ID><0x9958><phone number><GPS/Cell data><checksum>\r\n |
| 下行 | @@<L><ID><0x9958><phone number><SMS data><checksum>\r\n |
| 范例 | 2424 0086 30060000000007 9958 2B383631333438383635353138360000 3039323530372E3030302C412C323233322E323338302C4E2C31313430312E333739342C452C3030302E302C3139372E33372C3132303931342C2C7C31307C3130307C33393036342C393533332C3031302C3436307C303030307C30307C3232337C313030 2264 0D0A |
|  | 4040 010B 30060000000007 9958 2B383631333438383635353138360000 330030003000360030003000300030003000300030003000300037003A007F5E1C4E0177F16D3357025E8F7930753A53668F6C51995E2E00DD8DBB79F06C3671D1798062ED563300300036000B68320032003300737C2E00DD8DBB79F06C3671E55D388DED563200310036000B68320033003600737C2E00DD8DBB79F06C3671D1798062ED563300300034000B68320036003500737C2E00DD8DBB79E5651A900D54668FF47EEE4F33003200737C2E00DD8DBB79F16D3357025EB352B75F29527D6C668F0D67A152096750966C51F85333003300737C2E00DD8DBB79B073E34EA8534A5733003800737C EF24 0D0A |

注意： 1、“phone number”为16位的ASCII电话号码。 2、“GPS/Cell data”地址信息格式参考附录2。 3、“SMS data”为大端unicode码。 范例中地址信息：30030003000360030003000300030003000300030003000300037003A007F5E1C4E0177F16D3357025E8F7930753A53668F6C51995E2E00DD8DBB79F06C3671D1798062ED563300300036000B68320032003300737C2E00DD8DBB79F06C3671E55D388DED563200310036000B68320033003600737C2E00DD8DBB79F06C3671D1798062ED563300300034000B68320036003500737C2E00DD8DBB79E5651A900D54668FF47EEE4F33003200737C2E00DD8DBB79F16D3357025EB352B75F29527D6C668F0D67A152096750966C51F85333003300737C2E00DD8DBB79B073E34EA8534A5733003800737C 对应解码为字符： 30060000000007:广东省深圳市福田区车公庙.距离泰然科技园306栋226米~~~

3. 附录1: 0x9955数据格式

Data consists of: GPRMC | HDOP | Altitude | Cell | State | Alarm | Battery | Signal  For example: 100008.000,A,2232.4679,N,11356.7805,E,0.204,89.22,210911,,|7.49|152.6|3571,9763,00,460|0000|00|100|100

- (1) GPRMC includes: hhmmss.dd,S,xxmm.dddd,<N|S>,yyymm.dddd,<E|W>,s.s,h.h,ddmmyy

若GPS未定位，则缺省该字段。GPS定位时，数据包解析如下： 134829.486,A,1126.6639,S,11133.3299,W,58.31,309.62,110200

| Parameter | 描述 | Example |
| --- | --- | --- |
| hhmmss.dd | UTC 时间 hh = hours; mm = minutes; ss = seconds; dd = decimal part of seconds | 13:48:29.486 |
| S | Status indicator, A = valid, V = invalid | A=Valid |
| xxmm.dddd | Latitude xx = degrees; mm = minutes; dddd = decimal part of minutes | 11 deg. 26.6639 min |
| <N¦S> | Either character N or character S N = North, S = South | S = South |
| yyymm.dddd | Longitude yyy = degrees; mm = minutes; dddd = decimal part of minutes | 111 deg. 33.3299 min |
| <E¦W> | Either character E or character W E = East, W = West | W=West |
| s.s | Speed, knots | 58.31 Knots |
| h.h | Heading | 309.62 deg |

| ddmmyy | Date dd = date; mm = month‟ yy = year | 11th, Aug. 2000 |
| --- | --- | --- |

(2) | 分割符 ASCII (=0x7c) (3) HDOP, in ASCII code, 0.5-99.9. HDOP is blank when the tracking unit has no GPS fix (4) Altitude, in algorism. (5) Cell  includes: cid,lac,mnc,mcc  For example:4272,10147,00,460

| Parameter | Description | Example |
| --- | --- | --- |
| cid | Cell ID | 4272 |
| lac | LAC | 10147 |
| mnc | MNC | 00 |
| mcc | MCC | 460 |

(6) State: Status of input and output Bit0~Bit7: Reserved and default as „0‟  Bit8: Status of Input1  If Bit8=0: Input1 is inactive; Bit8=1: Input1 is active (SOS button is pressed)  Bit9: Status of Input2  If Bit9=0: Input2 is inactive; Bit9=1: Input2 is active (Button 1 is pressed)  Bit10: Status of Input3  If Bit10=0: Input3 is inactive; Bit10=1: Input3 is active (Button 2 is pressed)  Bit11: Status of Input4  If Bit11=0: Input4 is inactive; Bit11=1: Input4 is active (Button 3 is pressed) Bit12: Status of Input5  If Bit12=0: Input5 is inactive; Bit12=1: Input5 is active (Answer button is pressed)  Bit13: Status of Input6  If Bit13=0: Input6 is inactive; Bit13=1: Input6 is active (“Hang up/Power key” button is pressed)

(7) Alarm: Alarm command

| Alarm code | Definition |
| --- | --- |
| =0x01 | SOS button is pressed |
| =0x02 | 2nd button is pressed |
| =0x03 | 3rd button is pressed |
| =0x04 | First button is pressed |
| =0x05 | Reserved |
| =0x10 | Low battery alarm |
| =0x11 | Over speed alarm |
| =0x12 | Movement alarm |
| =0x13 | Reserved |
| =0x14 | Alarm of tracking unit being turned on |
| =0x15 | Alarm of tracking unit entering blind area |
| =0x16 | Alarm of tracking unit leaving blind area |
| =0x31 | SOS button is released |
| =0x32 | 2nd button is released |
| =0x33 | 3rd button is released |
| =0x34 | First button is released |
| =0x35 | Reserved |
| =0x40 | Vibrate alarm |

(8) Battery: Percentage of the battery voltage  (9) Signal: Percentage of the signal strength